# TPS610891 Boost Converter
Compensation Calculations

Provisional design for 5.0 V input, 5.8 V output, 100 mA load

Purpose. This document records the initial small-signal and compensation calculations for the TPS610891 stage that feeds an LT3045. The values are provisional until the exact output-capacitor part number, DC-bias derating, ESR, inductor, and measured loop response are confirmed. The calculation sequence follows the Texas Instruments TPS61089/TPS610891 datasheet, Equations 12–19.

# 1. Design Inputs and Assumptions

| Parameter | Value | Reason / source |
| --- | --- | --- |
| Input voltage, VIN | 5.0 V | USB-C 5 V nominal |
| Boost output voltage, VOUT | 5.8 V | Provides headroom for LT3045 regulating to 5.0 V |
| Design output current, IOUT | 0.10 A | 5VA load plus regulator current and margin |
| Switching frequency, fSW | 2.0 MHz | Selected design target |
| Inductor, L | 2.2 µH | Initial selected value |
| Assumed efficiency, η | 0.85 | Conservative provisional estimate |
| Effective output capacitance, CO | 45 µF | Assumed after DC-bias derating of 3 × 22 µF ceramics |
| Equivalent output-capacitor ESR, RESR | 3 mΩ | Provisional parallel equivalent |
| FB reference voltage, VREF | 1.212 V | TPS610891 typical PWM reference |
| Error-amplifier transconductance, GEA | 190 µS | TPS610891 typical value |
| Equivalent current-sense resistance, Rsense | 0.08 Ω | TPS610891 datasheet model value |

# 2. Output Load Resistance

The small-signal model uses the output load resistance:

Rₒ = VOUT / IOUT = 5.8 V / 0.10 A = 58 Ω

Result: Rₒ = 58 Ω.

# 3. Effective Duty Cycle

Using the datasheet expression that includes assumed conversion efficiency:

D = 1 − (VIN × η / VOUT)

D = 1 − (5.0 × 0.85 / 5.8) = 0.267

Result: D ≈ 0.267, or 26.7%.

# 4. Output Pole

fP = 2 / (2π × Rₒ × Cₒ)

fP = 2 / (2π × 58 × 45 µF) ≈ 122 Hz

Result: fP ≈ 122 Hz.

# 5. Capacitor ESR Zero

fESRZ = 1 / (2π × RESR × Cₒ)

fESRZ = 1 / (2π × 0.003 × 45 µF) ≈ 1.18 MHz

Result: fESRZ ≈ 1.18 MHz. The high value is expected with low-ESR ceramic capacitors.

# 6. Right-Half-Plane Zero

fRHPZ = Rₒ × (1 − D)² / (2π × L)

fRHPZ = 58 × (1 − 0.267)² / (2π × 2.2 µH) ≈ 2.25 MHz

Result: fRHPZ ≈ 2.25 MHz.

# 7. Crossover Frequency Selection

The datasheet recommends keeping crossover below the smaller of one-tenth of the switching frequency and one-fifth of the RHP zero:

fC ≤ min(fSW / 10, fRHPZ / 5)

fSW / 10 = 200 kHz

fRHPZ / 5 ≈ 451 kHz

Selected provisional fC = 200 kHz

Caution: 200 kHz is the maximum value allowed by this simple guideline, not a guaranteed optimum. A more conservative first-pass target such as 100–150 kHz may provide more phase margin and should be considered during simulation or bench verification.

# 8. Compensation Resistor R5

R5 = [2π × VOUT × Rsense × fC × Cₒ] / [(1 − D) × VREF × GEA]

R5 = [2π × 5.8 × 0.08 × 200 kHz × 45 µF] / [0.733 × 1.212 × 190 µS]

R5 ≈ 155.5 kΩ

Practical first-pass choice: 154 kΩ or 158 kΩ, 1%.

# 9. Compensation Capacitor C5

C5 = Rₒ × Cₒ / (2 × R5)

C5 = 58 × 45 µF / (2 × 155.5 kΩ) ≈ 8.39 nF

Practical first-pass choice: 8.2 nF. Use a stable dielectric where practical.

# 10. High-Frequency Capacitor C6

C6 = RESR × Cₒ / R5

C6 = 0.003 × 45 µF / 155.5 kΩ ≈ 0.87 pF

Because the calculated value is below 10 pF, the datasheet permits C6 to be left open. Keep a footprint and mark it DNP for tuning.

# 11. Provisional Component Values

| Item | Calculated value | Provisional schematic value |
| --- | --- | --- |
| R5 | 155.5 kΩ | 154 kΩ or 158 kΩ, 1% |
| C5 | 8.39 nF | 8.2 nF |
| C6 | 0.87 pF | DNP / open; retain footprint |
| Selected crossover | 200 kHz | Provisional upper-limit design |

# 12. Crossover-Frequency Sensitivity

Because R5 scales with crossover frequency and C5 changes inversely, the following alternatives are useful during review:

| fC | R5 calculated | C5 calculated | C6 calculated |
| --- | --- | --- | --- |
| 100 kHz | 77.75 kΩ | 16.78 nF | 1.74 pF |
| 150 kHz | 116.62 kΩ | 11.19 nF | 1.16 pF |
| 200 kHz | 155.50 kΩ | 8.39 nF | 0.87 pF |

# 13. Required Validation Before PCB Release

• Choose the exact 22 µF capacitor part number and obtain effective capacitance at 5.8 V from the manufacturer DC-bias curve.

• Use the exact equivalent ESR of the populated parallel capacitor bank.

• Confirm the exact 2.2 µH inductor, including tolerance and inductance reduction under bias.

• Recalculate all values if CO, RESR, L, efficiency, switching frequency, or load changes.

• Verify loop stability using TI WEBENCH, a suitable small-signal simulation, or a measured Bode plot.

• Confirm greater than 45° phase margin and greater than 10 dB gain margin, as recommended by the datasheet.

• Place R5, C5, and the optional C6 immediately beside the COMP pin with a short, quiet ground return.