Documentation:

Power rating for 5VA:

Chosen regulator: LT3045xDD, MAx output 600ma

Current consumed by 5VA rails:

| Device | Typical Current |
| --- | --- |
| AK4499EX VDDL + VDDR | 18 mA |
| AK4499EX VREFHL + VREFHR | 46 mA |
| AK4499EX AVDD | 3 mA |
| Subtotal | 67 mA |
| LT3045 ground current | ~3 mA |
| Boost converter output current Worst caseTPS610891 | 70 mA 0.18mA |
| 30% design margin | ≈91 mA |
| Design target | 100 mA |

Worst caseTPS610891                       0.18mA

Power rating for +15V and -15V rails:

| Device | Typical Current |
| --- | --- |
| OPA1622 | 6.76 mA(2 channels) |
| OPA1612 | 18mA |
| 30% design margin | ≈24.76mA |
| Design target | 25mA |

### 1. Component Current Breakdown (3.3V Rail)

| Component / Function | Parameter / State | Peak Current (Imax​) |
| --- | --- | --- |
| ESP32-S3-MINI-1U (U4) | Peak Wi-Fi/BT TX Transients (Assumption) | 355 mA |
| AK4191EQ Modulator (U1) | Digital DVDD / TVDD2 at 3.3V | ~5 mA |
| Master Clock Oscillators (Y1, Y2) | Active XO32 Crystal Oscillators | ~20 mA total |
| 74LVC1G3157 MUX (U5A) | Static + Switching Current | ~2 mA |
| Pull-Up Resistors & Passive Leakage | R9, R53, R7, R8, R13, R63 | ~3 mA |
| Total Peak Current | Worst-Case Simultaneous Draw | 385 mA |

### 1. Component Current Breakdown (1.2V Rail)

| Parameter / Operational Mode | Typical Current (Ityp​) | Max / Peak Current (Imax​) |
| --- | --- | --- |
| AK4191EQ DVDD (1.2V Core) | ~18 mA(PDF) | ~25 mA(PDF) |

# Power Budget

The purpose of the power budget is to estimate the maximum current required by each voltage rail, verify regulator sizing, and determine the total current ultimately required from the 5 V USB-C input.

The board contains the following main rails:

USB-C 5 V

│

├── TPS610891 → 5.8 V → LT3045 → 5VA

│

├── TPS65131 → +15 V / -15 V

│

├── 3.3 V regulator → ESP32-S3 + digital circuitry

│

└── AP2112K-1.2 → 1.2 V digital core

# 1. 5VA Analog Rail

### Regulator

LT3045 ultra-low-noise LDO

VOUT​=5.0V

Maximum regulator output capability:

IOUT,max​=500mA

The LT3045 is supplied from approximately 5.8 V generated by the TPS610891 boost converter.

## AK4499EX 5VA loads

| Device / Supply Domain | Typical Current |
| --- | --- |
| AK4499EX VDDL + VDDR | 18 mA |
| AK4499EX VREFHL + VREFHR | 46 mA |
| AK4499EX AVDD | 3 mA |
| AK4499EX subtotal | 67 mA |

Therefore:

I5VA,load​≈67mA

Add approximately 3 mA provisionally for LT3045 operating/ground current:

ILT3045,in​≈67+3 ILT3045,in​≈70mA

The TPS610891 also consumes approximately 180 µA maximum from its VOUT pin under the specified no-load condition.

70mA+0.18mA=70.18mA

### 30% design margin

Idesign​=70.18mA×1.30 Idesign​≈91.2mA

Round upward:

I5VA,design​=100mA​

### Result

| Parameter | Value |
| --- | --- |
| Estimated operating current | ~70 mA |
| Current with 30% margin | ~91 mA |
| Design target | 100 mA |
| LT3045 capability | 500 mA |

The LT3045 therefore has a substantial current margin.

# 2. TPS610891 5 V Input Requirement

The TPS610891 supplies approximately:

VOUT​=5.8V

at a design current of:

IOUT​=100mA

Assume:

ηboost​=85%

Input current is estimated from conservation of power:

IIN​=VIN​ηVOUT​IOUT​​ IIN​=5(0.85)5.8(0.1)​ I5V,TPS610891​≈136mA​

This 136 mA, rather than the converter's multi-amp switch rating, is what belongs in the overall USB power budget.

# 3. +15 V and −15 V Analog Rails

The ±15 V rails power the analog I/V and headphone amplifier stages.

Current-consuming devices:

| Device | Supply Current |
| --- | --- |
| OPA1612 I/V stages | 18 mA |
| OPA1622 stereo amplifier | 6.76 mA |
| Quiescent subtotal | 24.76 mA per rail |

However, this is only the idle amplifier current. The headphone load must also be included.

## Headphone load

Target headphone:

Sennheiser HD600 class

Nominal impedance:

RHP​=300Ω

Maximum intended output:

VHP​=9.2VRMS​

RMS headphone current per channel:

IHP,RMS​=RHP​VRMS​​ IHP,RMS​=3009.2​ IHP,RMS​≈30.7mA​

Peak headphone current:

IHP,peak​=2​IHP,RMS​ IHP,peak​=1.414(30.7) IHP,peak​≈43.4mA​

For a sinusoidal class-AB output stage, approximate average load current drawn from each supply rail per channel:

Irail,load​≈πIpeak​​ Irail,load​≈π43.4​ Irail,load​≈13.8mA

Stereo:

Irail,load,stereo​=2(13.8) Irail,load,stereo​≈27.6mA​

Now add quiescent amplifier current:

I+15​=24.76+27.6 I+15​≈52.36mA

Similarly:

I−15​≈52.36mA

### Add 30% design margin

52.36×1.30=68.1mA

Round upward:

I+15,design​=75mA​ I−15,design​=75mA​

### ±15 V summary

| Parameter | +15 V | −15 V |
| --- | --- | --- |
| Amplifier quiescent current | 24.76 mA | 24.76 mA |
| Stereo headphone contribution | ~27.6 mA | ~27.6 mA |
| Estimated operating requirement | ~52.4 mA | ~52.4 mA |
| 30% margin | ~68.1 mA | ~68.1 mA |
| Design target | 75 mA | 75 mA |

# 4. TPS65131 5 V Input Requirement

The TPS65131 produces both ±15 V rails.

Maximum design output power:

P±15​=15I+15​+15I−15​

Using:

I+15​=I−15​=75mA P±15​=15(0.075)+15(0.075) P±15​=2.25W

Assume approximately 80% converter efficiency:

η=0.80

Required 5 V input power:

PIN​=0.802.25​ PIN​=2.8125W

Therefore:

I5V,TPS65131​=52.8125​ I5V,TPS65131​≈563mA​

Use approximately:

0.56A​

in the USB/5 V rail budget.

# 5. 3.3 V Rail

Your current component estimate is:

| Component | Maximum / Peak Current |
| --- | --- |
| ESP32-S3-MINI-1U | 355 mA |
| AK4191EQ 3.3 V supply domain(s) | ~5 mA |
| Master-clock oscillators | ~20 mA |
| 74LVC1G3157 | ~2 mA |
| Pull-ups / leakage | ~3 mA |
| Total | 385 mA |

One terminology correction: do not label the AK4191 3.3 V entry DVDD if DVDD is the 1.2 V core rail in your design. Label the exact 3.3 V supply pin(s) so DVDD is not accidentally counted twice.

Because the ESP32 can produce short current transients, use a larger margin than the analog rails.

Use 50%:

I3.3V,design​=385mA×1.5 I3.3V,design​=577.5mA

Round upward:

I3.3V,design​=600mA​

If the 1.2 V regulator is powered from the 3.3 V rail, its input current must also be added here. In that case, use approximately:

600mA+40mA≈640mA

as the conservative 3.3 V source requirement.

# 6. 1.2 V Rail

The AK4191EQ core consumes:

| Parameter | Typical | Maximum |
| --- | --- | --- |
| AK4191EQ DVDD | ~18 mA | ~25 mA |

Use the maximum current:

I1.2,max​=25mA

Add 30%:

I1.2,design​=25×1.30 I1.2,design​=32.5mA

Round upward:

I1.2V,design​=40mA​

This is far below the AP2112K's available output-current capability.

### Important upstream accounting

If:

3.3 V → AP2112K-1.2 → 1.2 V

then the approximately 40 mA AP2112 input current is already part of the 3.3 V regulator load.

Do not add it to USB current a second time.

If instead:

5 V → AP2112K-1.2 → 1.2 V

then approximately 40 mA should be included directly in the 5 V budget.

# 7. Relay / Direct 5 V Loads

Your mute relay is another direct load.

Find its coil resistance or rated coil current from its datasheet.

If the coil resistance is given:

Irelay​=Rcoil​Vcoil​​

Example:

V=5V,R=167Ω Irelay​≈30mA

Enter the actual relay value when the part is finalized.

# 8. Total USB-C 5 V Budget

Using the numbers currently established:

| 5 V Load | Design Input Current |
| --- | --- |
| TPS610891 → 5VA system | ~136 mA |
| TPS65131 → ±15 V | ~563 mA |
| 3.3 V regulator | ~600 mA |
| 1.2 V regulator if directly from 5 V | ~40 mA |
| Relay | TBD |
| Other direct 5 V loads | TBD |

Ignoring the relay temporarily:

IUSB​=0.136+0.563+0.600+0.040 IUSB​≈1.339A

If the AP2112 is powered from 3.3 V instead, don't add its 40 mA separately; it is already included upstream.

So your current estimated full-load USB requirement is roughly:

1.3–1.4A​

before the relay and miscellaneous loads.

Add approximately 20% overall system margin:

1.34×1.20≈1.61A

Therefore I would design the USB-C power input around at least:

5V, 2A​

rather than assuming 4–6 A.

# Final Power Budget Summary

| Rail | Estimated / Peak Load | Design Margin | Design Target |
| --- | --- | --- | --- |
| 5VA | ~70 mA | 30% | 100 mA |
| +15 V | ~52.4 mA | 30% | 75 mA |
| −15 V | ~52.4 mA | 30% | 75 mA |
| 3.3 V | 385 mA | 50% | 600 mA |
| 1.2 V | 25 mA | 30% | 40 mA |
| USB-C 5 V | ~1.34 A + relay | ~20% system margin | ≈2 A source recommended |

| Net Class | Current | Via Diameter | Drill |
| --- | --- | --- | --- |
| Digital (GPIO, I²C, I²S) | <20 mA | 0.60 mm | 0.30 mm |
| Analog signals | <20 mA | 0.60 mm | 0.30 mm |
| 1.2 V rail | ~40 mA | 0.60 mm | 0.30 mm |
| 5VA rail | ~100 mA | 0.60 mm | 0.30 mm |
| +15 V / −15 V rails | ~75 mA | 0.60 mm | 0.30 mm |
| ESP32 3.3 V branch | ~355 mA | 0.80 mm | 0.40 mm |
| USB 5 V main input | 1–2 A | 1.00 mm | 0.50 mm or two 0.60/0.30 mm vias in parallel |
| Ground stitching | — | 0.60 mm | 0.30 mm |

| Net Class | Clearance | Track Width | Via Size | Via Hole | Used For |
| --- | --- | --- | --- | --- | --- |
| Signal | 0.20 mm | 0.20 mm | 0.60 mm | 0.30 mm | GPIO, I²C, control signals |
| Clock | 0.20 mm | 0.20 mm | 0.60 mm | 0.30 mm | MCLK, BCLK, LRCK |
| I2S | 0.20 mm | 0.20 mm | 0.60 mm | 0.30 mm | SDIN, SDOUT |
| USB_Power | 0.30 mm | 1.20 mm | 1.00 mm | 0.50 mm | USB-C 5V input |
| 5VA | 0.20 mm | 0.50 mm | 0.80 mm | 0.40 mm | LT3045 analog rail |
| 3V3_Main | 0.20 mm | 0.80 mm | 0.80 mm | 0.40 mm | Main 3.3V trunk |
| 3V3_Branch | 0.20 mm | 0.30 mm | 0.60 mm | 0.30 mm | Oscillators, AK4191, logic |
| 1V2 | 0.20 mm | 0.30 mm | 0.60 mm | 0.30 mm | AK4191 core |
| +15V | 0.20 mm | 0.50 mm | 0.80 mm | 0.40 mm | Analog amplifier supply |
| -15V | 0.20 mm | 0.50 mm | 0.80 mm | 0.40 mm | Analog amplifier supply |
| HP_OUT | 0.20 mm | 0.80 mm | 0.80 mm | 0.40 mm | Left/Right headphone outputs |
| AGND | — | Plane | 0.60 mm | 0.30 mm | Analog ground plane |
| DGND | — | Plane | 0.60 mm | 0.30 mm | Digital ground plane |

Rule of thumb design rules:

:

| Area | Suggested placement |
| --- | --- |
| General pull-ups / GPIO resistors | 1–3 mm apart is fine |
| I²C pull-ups | No critical spacing; keep near the bus/controller |
| OPA1612 I/V feedback parts | Very close to op-amp, ~1–2 mm if possible |
| OPA1622 differential/summing resistors | Close together and close to op-amp |
| LPF resistors/capacitors | Keep the whole filter cluster compact |
| TPS610891 FB/COMP resistors | Very close to FB/COMP pins, away from SW/inductor |
| LT3045 SET/PGFB resistors | Close to LT3045, referenced to quiet ground |
| Relay/output resistors | Can be several mm apart; not noise-critical |

| Capacitor type | Distance from IC pin |
| --- | --- |
| 100 nF high-frequency decoupling | 1–3 mm ideally |
| 1 µF local decoupling | 2–5 mm |
| 10–22 µF bulk/local rail capacitor | 5–15 mm is usually fine |
| Switching regulator input/output caps | as close as physically possible |